| Airport, runways, TPA, frequencies To | CH | Distance |  | Est’ GS | Est’ Time |  | Fuel: |  | Actual Time: |  |  | To |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Leg | Total |  | Leg | Total | Leg | Total | Leg | Total |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Wind speed |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Wind dir |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | TC |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R+L-WCA |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | = TH |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | W+E-Var |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | =MH |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | DEV |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | CH |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | TAS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | GS |  |  |

| station | 3000 | 6000 | 9000 | 12000 |
| --- | --- | --- | --- | --- |

PIREP Sequence

- Location in reference to a NAVAID, airport, or a significant geographical landmark.

- Altitude.

- type of aircraft.

- Sky cover including amount (FEW, SCT, BKN, OVC).

- bases and tops.

- Flight visibility and weather encountered.

- Outside air temperature (OAT).

- Observed winds aloft.

- Turbulence and icing.

- Other significant weather data(e.g., distant weather observed)

Weather

Adverse conditions, synopsis, current conditions, en route forecast, destination forecast, winds aloft (above), NOTAMs

Flight Plan Sequence

- VFR or IFR.

- Registration.

- Type & Equipment.

- TAS in Knots.

- Departure point.

- Departure time.

- Altitude.

- Route.

- Destination.

- Estimated time en route.

- Remarks.

- Fuel time.

- Alternate.

- Name, contact phone, home base airport.

- Number of people.

- Colors.